# Progress Note — Shovel Load-Torque Project

Project IIT/SRIC/R/AEH/2026/104 · generated 2026-07-11 from research_log.md · JRF: Suhail Majeed Sheikh

# 2026-07-07 (session 2) — STAGE 1 CLOSED

What was done:: Wired the two acceleration ports (revolute 'b' → q_3_ddot,

prismatic 'a' → d_4_ddot, each via PS-Simulink → To Workspace). Ran

validate_and_identify_4 with sensed accelerations.

Key result (stage1_results.mat, archived in experiments/2026-07-07_stage1_final/)::

- Revolute equation:  R² = 1.000000, RMSE = 0 Nm

- Prismatic (crowd) equation (validated for the FIRST time): R² = 1.000000, RMSE = 0 N

- Identification: m=54,300.0±0.0 | c=−1.3200±0.0000 | Izz=287,900±0 |

fv3=3,800.0±0.0 | fc3=0.0±0.0 — every parameter exact, machine precision.

- Residuals at 1e−13 scale (floating-point floor).

Interpretation (honest version for the thesis):: with noise-free sensed

signals, the hand-derived equations are term-for-term identical to the

multibody solution; ALL previous residual (0.79% NRMSE, spikes at velocity

zero-crossings, 56% Izz bias) was numerical differentiation error. Zero-width

CIs are a simulation-only phenomenon; field data will reopen them — expected.

Status vs objective:: objective NOT yet achieved — Stage 1 built and proved

the instrument (a model whose residual channel is clean to 1e−13). Novelty

begins at Phase C.

Next step (Phase C):: inject a known time-varying external force at the

dipper in Simscape (unknown to the analytical model); demonstrate

τ_load(t) = τ_meas − Y·Φ̂ recovers it. Then Phase D: momentum observer

(no q̈ needed → deployable on real machines).

Also produced:: CMPDI data-requirements document (meetings/) for the

2026-07-08 Coal India meeting — shovel phase only.

---

## TEMPLATE — copy for each new session

## YYYY-MM-DD — <one-line title>

What was done::

Key result::

Decisions & why::

Next step::

Open questions::

# 2026-07-07 (session 1) — External review + file consolidation

What happened:: Independent review (Gemini) of the v3 script found 4 valid

issues: (1) stale comments contradicting the model, (2) isfield/isprop on

SimulationOutput always false → sensed accelerations never used, (3) friction

missing from τ_pred, (4) crowd equation logged but never validated.

Verification of the review (do not skip this step for ANY external file)::

- Confirmed all four issues real.

- Confirmed their G7: logged q_d is in rad/s (q plotted ×180/π gave degrees),

so θ4 is already N·m·s/rad; my old ×180/π printout was wrong.

- Confirmed their G8: the Rasuli paper contains NO "R²=0.942" anywhere —

a citation I (Claude session) had fabricated from bad memory. Paper actually

reports: payload 8,402 vs 8,420 kg actual, σ=104 kg (swing);

8,340 kg, σ=579 kg (lifting). Struck the false number everywhere.

- Found one bug in THEIR file: corr() needs Statistics Toolbox → replaced

with corrcoef() (base MATLAB).

Adopted:: shovel_params.m as single source of truth (all numbers, one file)

+ validate_and_identify_4.m (fixed). HPINN paper (Fu 2022) re-read: confirms

R²/MAE/RMSE are the field-standard metrics (their Eq. 18) and its hybrid loss

J = α·J_lagrangian + η·J_energy is the Stage-4 blueprint.

---

# 2026-07-06 (session 2) — The axis bug hunt (three geometry bugs found)

Symptom:: Measured torque only ±35 kNm; expected ~4.4 MNm from m·g·r alone.

R² catastrophically negative. Identification returned negative mass/Izz.

Diagnosis chain (keep this — it is a thesis-methodology exhibit)::

- g=0 test: torque UNCHANGED with gravity off ⇒ gravity contributed nothing.

- Torque waveform at 2× excitation frequency ⇒ pure Coriolis signature

(velocity×velocity products double frequency).

- ⇒ Revolute spin axis was VERTICAL (Simscape default Z ∥ gravity): the

mechanism was a horizontal carousel; gravity has no moment about a

vertical axis. Confirmed by model tree label "Rz".

- After first fix attempt, identification "fit perfectly with wrong params":

R²_fit=0.9999 with effective m·r = 70,672 kg·m ⇒ r_eff ≈ 1.30 m, CONSTANT.

Only constant ~1.3 m in the model: |1.5 − 2.82| = 1.32 m ⇒ the prismatic

joint was sliding ALONG the spin axis (axial), not radially. Lesson:

when least squares fits perfectly with wrong parameters, it is telling

you the true geometry of the system actually built.

Fix (model v4, "tilt gravity not the mechanism")::

- Rigid Transform1 rotation reverted to None.

- Middle Rigid Transform: rotation StandardAxis +Y 90° → slide made radial.

- Dipper COM moved to [0 0 −2.82] (along slide axis); inertia to [Izz 0 0]

(axis parallel to spin).

- Gravity set in-plane: [0 −9.80665 0]. Lever arm becomes r = d4 − 1.32.

- One remaining sign flip in sensed torque fixed in script (tau = +out.tau_hoist).

Parameter provenance decision:: replaced the arbitrary built-in body values

(90,300 / −5 / 839,000 — traceable to NO paper; checked Frimpong 2004/2006/2008

and all project docs) with Rasuli 2014 Table II (dynamic case):

Md=54,300 kg, L_COG=2.82 m, Izz3=287,900 kg·m², fv3=3,800 N·m·s/rad,

fc3=340 N·m, fv4=1,473 N·s/m, fc4=480 N. NOTE: I initially misread Table II

(swapped fv3/fc3 columns); caught by checking the rendered table against the

plain-text extraction. Always verify table reads against the PDF image.

Excitation fix:: Sine Wave1 (d4) amplitude 0.25→2 m; cond(Y) scaled

dropped from ~10^4–10^5 territory to 87 (healthy).

---

# 2026-07-06 (session 1) — Model dissection via raw .slx XML

What was done:: Unpacked ShovelSimulator.slx (it is a zip of XML) and read

every block parameter directly, instead of trusting assumptions.

Findings (all verified in XML, not from memory)::

1. Solver = VariableStepAuto → the script's linspace() time vector was WRONG.

Fix: always use out.tout.

2. Dipper Handle brick: mass 90,300 kg, COM [−5 0 0], custom Izz = 839,000 kg·m²

→ the point-mass formula was missing the Izz·q̈ parallel-axis term.

3. Rigid Transform between joints carries a 1.5 m translation → lever arm is

not simply d−5.

4. Simulink-PS input converters filter with 2nd-order, τ=0.05 s → startup

transient at t<0.5 s is expected, must be trimmed.

5. Revolute joint already had SenseAcceleration=on (unused 'b' port).

Decisions:: Rewrote script (validate_and_identify.m) with: tout time base,

gradient() fallback for accelerations, Izz term, startup trim, and the

linear-in-parameters identification Y·θ=τ with θ=[m, mc, mc²+Izz]

(Tafazoli-1999/Rasuli-2014 regrouping trick).

---

# 2026-07-04 — First forward validation attempt (pre-log, reconstructed)

What was done:: Built 2-DOF Simscape model (ShovelSimulator.slx): revolute

saddle joint q3 + prismatic crowd d4, sine-driven motion, sensed tau_hoist.

Wrote first validation script comparing Simscape torque against hand-derived

point-mass Euler-Lagrange formula: τ = m(d_eff²·q̈ + 2·d_eff·ḋ·q̇ + g·d_eff·cos q).

Key result:: Curves tracked in shape but with systematic ~9–18% offset;

predicted above measured everywhere.

Next step:: Root-cause the offset instead of accepting "roughly matches."

---