Data and Support Requirements from CMPDI / Coal India Limited

Electric Rope Shovel Phase — Physics-Based Load-Torque Estimation and Health Monitoring

Project No. IIT/SRIC/R/AEH/2026/104 — AI-Enabled Platform for Diagnosis, Prognosis and Health Management of Mining Electrical Systems

Department of Mining Engineering, Indian Institute of Technology Kharagpur

Prepared for review meeting — 08 July 2026

# 1. Purpose of this document

This document consolidates, in one place, every item of information, data, and support the IIT Kharagpur project team requires from CMPDI / Coal India Limited to progress the electric rope shovel work package from its current simulation-validated stage to field-data validation on an operating machine. Each request is stated with the reason it is needed, the priority, and the preferred format, so that the concerned mine, workshop, and OEM-liaison teams can act on it without further clarification. The present phase concerns the electric rope shovel only; the dragline case study follows in a later phase and is not covered by the requests below.

# 2. Current project status (context for the requests)

The team has completed Stage 1 of the technical roadmap: a two-degree-of-freedom dynamic model of the shovel front end (saddle-block pivot and crowd extension) has been derived from first principles using Euler-Lagrange mechanics and verified against an independent multibody simulation (MATLAB Simscape). Both governing equations — the saddle (hoist-side) torque equation and the crowd force equation — reproduce the simulated machine exactly, and all physical parameters (dipper-handle mass, centre-of-gravity offset, rotational inertia, and viscous friction coefficients) are recovered blind from motion and torque data using a least-squares identification scheme, to machine precision on noise-free data. All model parameters are currently taken from the published literature (Rasuli, Tafazoli and Dunford, IEEE 2014, dynamic identification results for a P&H-class cable shovel).

The immediate next steps are: (a) demonstrating recovery of an unknown, time-varying external load torque from motor-side measurements in simulation, and (b) replacing literature parameters and simulated signals with the actual parameters and drive signals of a Coal India shovel. Step (b) is entirely dependent on the items requested below.

# 3. Section A — Machine identification and fleet information

Before any parameter or signal can be requested precisely, the specific machine must be identified. We request:

- Nomination of one (or more) candidate electric rope shovel(s) for the study: make, model, dipper capacity, year of commissioning, subsidiary, mine, and bench where deployed.

- Machine population overview: how many shovels of the same model exist across the subsidiary/company, since transferability of the method across a fleet is a stated project objective.

- Utilisation pattern of the nominated machine: shifts per day, typical cycles per shift, planned maintenance windows during which measurements or instrumentation would be feasible.

- OEM identity and the status of any OEM service/technology agreement, since some parameter and drive data may have to be routed through the OEM.

# 4. Section B — Mechanical and geometric parameters

The dynamic model requires the following machine-specific quantities. Where a value is not directly documented, OEM drawings or CAD from which we can compute it are an acceptable substitute (and are in fact preferred for inertia). Priorities: E = essential for the model to run at all; I = important for accuracy; D = desirable.

| Parameter | Why the model needs it | Priority | Preferred source |
| --- | --- | --- | --- |
| Dipper mass (empty) and dipper-handle mass, separately and combined | The combined mass Md is the single largest coefficient in both dynamic equations | E | OEM datasheet / weighbridge record |
| Centre of gravity of the dipper-handle assembly (distance from handle end or saddle pivot) | Sets the gravity lever arm; identified as the second most sensitive parameter | E | OEM drawing / CAD |
| Mass moment of inertia of dipper-handle about its COG | Inertia term in the saddle equation | I | CAD model (we compute) or OEM value |
| Saddle-block pivot location on the boom (dimensions from boom foot) | Defines the fixed geometric offset in the kinematic chain | E | General-arrangement drawing |
| Handle length and crowd travel range (min/max extension) | Defines the range of the prismatic coordinate d4 | E | OEM specification |
| Boom length, boom angle, boom-point sheave radius | Required for the hoist-rope geometry that maps rope force to joint torque | E | General-arrangement drawing |
| Crowd transmission details: rack-and-pinion or rope crowd; pinion pitch radius; total gear ratio motor-to-pinion | Maps crowd motor torque/current to force on the handle | E | OEM transmission drawing |
| Hoist transmission details: drum radius, total gear ratio, number of rope falls, rope diameter | Maps hoist motor torque/current to bar-pull at the dipper | E | OEM transmission drawing |
| Swing gear ratio and upper-works inertia | Needed for the later 4-DOF extension; not blocking now | D | OEM data |
| OEM-stated transmission efficiencies or friction data, if any | Improves the friction terms beyond literature values | D | OEM service documentation |

# 5. Section C — Electrical drive and motor data

The project estimates joint torques from motor electrical quantities; the accuracy of the entire method therefore rests on knowing the electromechanical conversion exactly. We request, for the hoist and crowd drives (swing desirable for the later phase):

- Drive architecture: DC (Ward-Leonard / thyristor) or AC variable-frequency. This decides whether torque is estimated from armature current and field, or read from the drive's internal torque estimate.

- Motor nameplate data: rated power, rated/base speed, rated armature current and voltage, rated torque.

- Torque constants (torque per ampere) for each motor, including their variation with field current if DC. Note: published work on a comparable machine shows the hoist torque constant changes materially between field-current operating points (e.g., lowering vs hoisting regimes), so the field-weakening schedule or field-current log is part of this request.

- Drive/PLC parameter list confirming which internal signals exist (torque reference, torque actual, speed actual, current actual) and at what resolution they can be exported.

- Single-line electrical schematic of the shovel drives, to plan any non-intrusive current measurement (split-core sensors) without interfering with protection circuits.

# 6. Section D — Existing instrumentation, monitoring and data systems

Much of what the project needs may already be logged. Before proposing any new instrumentation we request a clear picture of what exists:

- Inventory of on-board monitoring: OEM production/health monitoring systems fitted to the nominated shovel (if any), the signal list they record, and their sampling rates.

- Mine-side data infrastructure: SCADA/historian systems that receive shovel data, data retention period, and the export mechanism (CSV, OPC, database access).

- Fleet-management/dispatch system in use (if any) and whether per-pass or per-truck payload estimates from it can be exported with timestamps.

- Whether any encoder/resolver signals for hoist, crowd or swing motion are available at the drive or PLC level — direct motion measurement would materially simplify validation, but the method is designed to work without it.

- Contact details of the PLC/drive maintenance engineer for the nominated machine, for tag-list level discussions.

# 7. Section E — Operational time-series data (the core request)

This is the central data request of the project. We ask for time-synchronised recordings of the following signals from the nominated shovel, during normal production, covering both empty travel/swing and loaded digging cycles:

| Signal | Purpose in the model | Priority | Minimum rate |
| --- | --- | --- | --- |
| Hoist motor armature current (and voltage), or AC drive torque-actual | Primary input: hoist-side torque estimate | E | 30 Hz |
| Crowd motor armature current (and voltage), or AC drive torque-actual | Primary input: crowd-side force estimate | E | 30 Hz |
| Field currents of hoist and crowd motors (if DC) | Corrects the torque constant across operating regimes | E | 30 Hz |
| Motor speeds (hoist, crowd) from drive/encoder | Joint velocity reconstruction; cross-check of kinematics | I | 30 Hz |
| Swing motor current and speed | Later 4-DOF phase; log now if free to do so | D | 30 Hz |
| Drive status/fault flags, operator master references | Cycle segmentation (dig / swing / dump / return) | I | event/1 Hz |

Scope of recording: ideally several complete shifts (covering different operators, bench conditions and payloads), but even a few hours of clean, time-stamped data covering 50-100 complete dig-swing-dump cycles is enough to begin. A shorter pilot export of any historical data that already exists in the historian — even at lower sampling rates — would be valuable immediately, ahead of any dedicated recording campaign.

Format: any consistent, documented format is acceptable (CSV, MAT, historian export). Each file must carry absolute timestamps and a channel legend. If sampling rates differ across channels, that is workable provided each channel's rate is stated.

# 8. Section F — Ground-truth payload records for validation

To validate payload and load-torque estimates against reality, the recorded drive data must be matchable to independently known payloads:

- Truck weighbridge records (time-stamped) for trucks loaded by the nominated shovel during the recording window, with truck IDs.

- Dispatch-system per-truck or per-pass payload figures for the same window, if such a system is in use.

- If feasible during a maintenance window: one or two controlled tests where a known weight (weighed bucket load, or a load verified over the weighbridge immediately after) is handled through a full cycle. This replicates the known-load calibration step used in the published methodology the project builds on.

- Bucket fill-factor or material density assumptions used by the mine for production accounting, so we interpret dispatch figures correctly.

# 9. Section G — Maintenance, failure and reliability records

The health-management objective of the project needs the historical failure picture of the machine class, so that the eventual diagnostic indicators target failures that actually matter to Coal India:

- Breakdown/maintenance history of the nominated shovel (and, if available, of the same model across the subsidiary) for the past 3-5 years: component, failure mode, downtime, corrective action.

- The mine's current condition-monitoring practice for shovels (vibration rounds, oil analysis, motor testing) and any records thereof.

- A ranked list, from the maintenance team's experience, of the electrical/drive-train failure modes that cause the most downtime — this directly shapes which health indicators we prioritise.

# 10. Section H — Site access, instrumentation permission and safety

- Permission and scheduling for a site visit by the project team to the nominated mine and machine, including cab/machinery-house access during a maintenance window.

- Statement of the safety inductions, medical requirements and statutory clearances (including any DGMS considerations) the team must complete, with expected lead times.

- In-principle permission to install non-intrusive measurement apparatus if existing logging proves insufficient: split-core current sensors around motor conductors, and possibly a sensor unit on the saddle block. The published methodology this project extends used exactly such non-contact apparatus, installed without cutting or parting any cable.

- Identification of any restriction on photographing/measuring the machine, so drawings requests can be substituted where photography is not permitted.

# 11. Section I — Administrative and data governance

- A named nodal officer at CMPDI and at the operating subsidiary/mine for this work package, through whom data and access requests flow.

- The data-sharing mechanism and any confidentiality instrument CMPDI requires (the institute will comply with Coal India's format); clarity on what may appear in academic publications and what must remain anonymised.

- Preferred transfer channel for bulk time-series data (secure drive, physical media, or institutional transfer).

- Agreement on a review cadence (suggested: monthly progress note; quarterly review) and the acceptance criteria CMPDI wishes to apply to the field-validation milestone.

# 12. Consolidated checklist (one-page summary for circulation)

| # | Item requested | Priority | Suggested owner |
| --- | --- | --- | --- |
| A1 | Nominated shovel: model, capacity, mine, subsidiary | Essential | CMPDI / subsidiary planning |
| A2 | Fleet count of same model; utilisation and maintenance windows | Important | Subsidiary planning |
| B1 | Dipper and handle masses; COG location; inertia or CAD | Essential | OEM liaison / excavation dept. |
| B2 | Saddle pivot, boom, handle geometry (GA drawings) | Essential | OEM liaison |
| B3 | Hoist and crowd transmission data (drum/pinion radii, ratios) | Essential | OEM liaison / workshop |
| C1 | Drive architecture; motor nameplates; torque constants incl. field dependence | Essential | E&M department |
| C2 | Drive/PLC exportable signal list and schematic | Essential | E&M / PLC engineer |
| D1 | Inventory of existing monitoring, SCADA/historian, dispatch payloads | Essential | Mine E&M / IT |
| E1 | Time-series export: hoist & crowd currents/voltages, field currents, speeds, ~30 Hz, time-stamped | Essential | E&M / IT |
| E2 | Any existing historical drive data (pilot export) | Important | Mine IT |
| F1 | Weighbridge and dispatch payload records matched to recording window | Essential | Mine operations |
| F2 | One controlled known-load test during maintenance window | Important | Mine operations / workshop |
| G1 | 3-5 year breakdown and maintenance history of machine/class | Important | Maintenance planning |
| G2 | Ranked list of costliest drive-train failure modes | Important | Maintenance team |
| H1 | Site visit permission, safety induction requirements, lead times | Essential | Mine management |
| H2 | In-principle permission for non-intrusive sensors if needed | Important | Mine management / E&M |
| I1 | Nodal officers (CMPDI + mine); data-sharing instrument; transfer channel | Essential | CMPDI |
| I2 | Publication/anonymisation policy; review cadence | Important | CMPDI |

# 13. Closing note

Items marked Essential in Sections B, C and E are the minimum set required to move the validated model onto a real machine; everything else improves accuracy, robustness, or the later health-monitoring phase. The team is prepared to work with partial data immediately — in particular, any existing historian export, however imperfect, allows data-format, cycle-segmentation and torque-constant work to begin in parallel with the formal recording campaign. We request that owners and tentative dates be attached to each checklist item during the meeting.